SIMATS ENGINEERING

Department of Computer Science and Engineering ASSESSMENT TOOL 1 – EXPERIMENTS

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO3 – Precision (BL3) | Assessment: | Experiments |
| Weightage: | 45% | Total Marks: | 100 |
| CO3 Statement: | Design processor organization by constructing pipelined datapath structures, identifying hazard types (data, control, structural), and comparing superscalar techniques such as out-of-order execution, speculative execution, and simultaneous multithreading (SMT). |  |  |
| Student Name: | J. Lakshmi Devi | Reg. No.: | 192524300 |
| Section / Batch: | 2025 | Date: | 10.08.2026 |

Instructions to Students - Identify the relevant concepts of register transfers, ALU operations, bus organization, pipelining, hazards, and superscalar processing. - Analyze the execution of data transfers, arithmetic and logic operations, instruction processing, and parallel execution. - Apply suitable techniques such as multiple buses, pipelining, stalling, forwarding, branch prediction, and speculative execution. - Compute performance measures such as execution time, clock cycles, speedup, throughput, and resource utilization. - Interpret the results by evaluating performance improvements, bottlenecks, and the suitability of each architectural technique.

QUESTIONS

- Design and simulate a system that performs register transfer operations along with basic arithmetic and logic operations such as addition, subtraction, AND, and OR. Use multiple registers to demonstrate how data is transferred between them during execution, and analyze the sequence of operations and the role of the ALU in processing data.

1. Register Transfer, Arithmetic and Logic Operations

Aim

To design and simulate a system that performs register transfer operations and basic ALU operations such as addition, subtraction, AND, and OR using multiple registers.

Registers Used

- R1, R2 – Input registers

- R3 – Temporary register

- R4 – Result register

- ALU – Performs arithmetic and logic operations

Operation Sequence

| Step | Operation | Description |
| --- | --- | --- |
| 1 | R1 ← Data1 | Load first data |
| 2 | R2 ← Data2 | Load second data |
| 3 | R3 ← R1 | Transfer R1 to R3 |
| 4 | R4 ← R1 + R2 | Addition |
| 5 | R4 ← R1 − R2 | Subtraction |
| 6 | R4 ← R1 AND R2 | Logical AND |
| 7 | R4 ← R1 OR R2 | Logical OR |

Example

Let:

- R1 = 1010

- R2 = 1100

Results:

- Addition = 10110

- AND = 1000

- OR = 1110

Role of ALU

The ALU (Arithmetic Logic Unit) receives operands from the registers, performs the required arithmetic or logical operation, and sends the result back to the destination register.

Conclusion

The simulation demonstrates how registers transfer data through the datapath and how the ALU performs arithmetic and logical operations during instruction execution.

- Create a model of a computer system using single-bus and multi-bus organization, and perform data transfer operations between registers, memory, and I/O. Compare the time taken and efficiency of both approaches, and analyse how multiple bus structures improve parallel data transfer and reduce bottlenecks.

# 2. Single-Bus and Multi-Bus Organization

### Aim

To model and compare single-bus and multi-bus computer organization for data transfer between registers, memory, and I/O devices.

### Single-Bus Organization

In a single-bus system, all components share one common bus.

Example:

R1 → BUS → R2

For an addition:

R1 → BUS → ALU ← R2

Only one major transfer can occur at a time.

### Multi-Bus Organization

A multi-bus system uses multiple buses simultaneously.

Example:

R1 ──→ Bus A ──→ ALU ──→ Bus C ──→ R3
R2 ──→ Bus B ──→ ALU

Both operands can reach the ALU at the same time.

### Comparison

| Feature | Single Bus | Multi Bus |
| --- | --- | --- |
| Number of buses | 1 | 2 or more |
| Data transfer | Sequential | More parallel |
| Hardware cost | Low | Higher |
| Speed | Lower | Higher |
| Bottleneck | High | Reduced |
| Parallel operations | Limited | Better |
| Control complexity | Simple | More complex |

### Analysis

A single bus creates a bottleneck because different components compete for the same communication path. Multi-bus organization allows simultaneous transfers, reducing the number of clock cycles required.

### Conclusion

Multi-bus organization provides higher performance and better parallelism, while single-bus organization is simpler and less expensive.

- Simulate a 5-stage instruction pipeline (Fetch, Decode, Execute, Memory, Write-back) and execute a set of instructions. Measure the performance in terms of speedup and throughput, and compare it with a non-pipelined system to evaluate the effectiveness of pipelining.

3. Five-Stage Instruction Pipeline

Aim

To simulate a 5-stage instruction pipeline and compare its performance with a non-pipelined processor.

Pipeline Stages

- IF – Instruction Fetch

- ID – Instruction Decode

- EX – Execute

- MEM – Memory Access

- WB – Write Back

Example Instructions

I1: ADD R1, R2, R3
I2: SUB R4, R5, R6
I3: AND R7, R8, R9
I4: OR R10, R11, R12

Pipeline Execution

| Cycle | I1 | I2 | I3 | I4 |
| --- | --- | --- | --- | --- |
| 1 | IF |  |  |  |
| 2 | ID | IF |  |  |
| 3 | EX | ID | IF |  |
| 4 | MEM | EX | ID | IF |
| 5 | WB | MEM | EX | ID |
| 6 |  | WB | MEM | EX |
| 7 |  |  | WB | MEM |
| 8 |  |  |  | WB |

Performance

For n instructions and k pipeline stages:

Non-pipelined cycles:

Tnon​=n×k

Pipelined cycles:

Tpipe​=k+(n−1)

For 4 instructions and 5 stages:

Speedup

Conclusion

Pipelining significantly improves instruction throughput by overlapping different stages of multiple instructions.

- Develop a pipeline execution scenario where data hazards, control hazards, and structural hazards occur. Observe their impact on instruction execution, and implement techniques such as stalling, data forwarding, and branch prediction to resolve these hazards and improve performance.

4. Pipeline Hazards and Their Solutions

Aim

To study data, control, and structural hazards in a pipeline and implement techniques to reduce their effects.

1. Data Hazard

Example:

I1: ADD R1, R2, R3

I2: SUB R4, R1, R5

I2 needs the result produced by I1.

Solution:

- Stalling

- Data forwarding

2. Control Hazard

Example:

I1: BEQ R1, R2, LABEL

I2: ADD R3, R4, R5

The processor does not immediately know whether the branch will be taken.

Solution:

- Branch prediction

- Pipeline flushing

- Delayed branching

3. Structural Hazard

Occurs when two instructions require the same hardware resource simultaneously.

Example:

Instruction 1 → Memory

Instruction 2 → Memory

Solution:

- Add additional hardware resources

- Stall one instruction

- Use separate instruction and data caches

Hazard Comparison

| Hazard | Cause | Solution |
| --- | --- | --- |
| Data | Data dependency | Forwarding/Stalling |
| Control | Branch instructions | Branch prediction |
| Structural | Resource conflict | Extra resources/Stalling |

Performance Analysis

Without hazard handling:

Instructions → Stalls → Reduced throughput

With hazard handling:

Instructions → Forwarding/Prediction → Fewer stalls → Higher throughput

Conclusion

Hazards reduce pipeline efficiency. Forwarding, stalling, and branch prediction help maintain better instruction throughput.

- Analyse a processor supporting superscalar execution, including out-of-order and speculative execution, and extend the study to include hyper-threading and simultaneous multithreading (SMT). Evaluate how multiple instructions are executed in parallel and compare the performance improvements in terms of instruction throughput and resource utilization.

# 5. Superscalar, Out-of-Order and SMT Execution

### Aim

To analyse a processor capable of superscalar, out-of-order, speculative and simultaneous multithreaded execution.

### Superscalar Execution

A superscalar processor can execute multiple instructions in one clock cycle.

Example:

Cycle 1 → ADD | SUB | AND

Cycle 2 → OR  | MUL | LOAD

Instead of executing one instruction at a time, multiple independent instructions can execute simultaneously.

### Out-of-Order Execution

Instructions do not always execute in program order.

Example:

I1 → Waiting for data

I2 → Ready

I3 → Ready

The processor can execute I2 and I3 before I1 instead of leaving execution units idle.

### Speculative Execution

The processor predicts the likely execution path, especially for branches, and executes instructions before the branch result is confirmed.

### Hyper-Threading / SMT

Simultaneous Multithreading (SMT) allows multiple instruction streams or threads to share processor resources.

Example:

CPU Core

/            \

Thread 1        Thread 2

↓                ↓

Instructions     Instructions

\            /

Shared Resources

### Performance Comparison

| Architecture | Parallelism | Resource Utilization | Throughput |
| --- | --- | --- | --- |
| Scalar | Low | Low | Low |
| Pipelined | Medium | Better | Higher |
| Superscalar | High | High | Very High |
| Out-of-order | Higher | Very High | Higher |
| SMT | Multiple threads | Better utilization | Very High |

### Performance Measures

Instruction Throughput:

Speedup:

### Conclusion

Superscalar and out-of-order execution increase instruction-level parallelism, while speculative execution reduces branch-related delays. SMT improves utilization of processor resources by allowing multiple threads to execute simultaneously.

# Code of Conduct

I J. LAKSHMI DEVI (192524300) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the Student: _________________________

# RUBRICS FOR CALCULATION

| Criteria | Weightage(%) | Level 4 – Exemplary (5 Marks) | Level 3 – Proficient (4 Marks) | Level 2 – Developing (2–3 Marks) | Level 1 – Beginning (0–1 Mark) |
| --- | --- | --- | --- | --- | --- |
| Understanding of Concepts | 25 | Demonstrates thorough understanding and effectively integrates multiple concepts/technologies | Good understanding with proper integration of most concepts | Basic understanding; limited or partial integration | Poor understanding; unable to integrate concepts |
| Experimental Design | 30 | Well-planned, systematic implementation with optimal solution | Correct implementation with minor issues | Basic implementation with errors or inefficiencies | Incorrect or incomplete implementation |
| Use of Tools | 20 | Effective and advanced use of tools/technologies with proper configuration | Appropriate use of tools with correct execution | Limited or inconsistent use of tools | Incorrect or minimal use of tools |
| Analysis of Results | 15 | Insightful analysis with accurate interpretation and validation of results | Good analysis with reasonable interpretation | Basic analysis with limited insights | Poor or incorrect analysis of results |
| Documentation | 10 | Clear, well-structured documentation with diagrams, code, and results | Organized documentation with necessary details | Basic documentation with missing details | Poor or incomplete documentation |

Marks Evaluation:

| Q. No | Understanding of Concepts(25) |  | Experimental Design (30) | Use of Tools(20) | Analysis of Results (15) | Documentation (10) | Total (out of 100) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 2 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 3 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 4 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 5 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
|  | / 25 |  | / 30 | / 20 | / 15 | / 10 | / 100 |

# Course Coordinator  	 	              	 	 	Faculty Incharge