CSA1220 – Computer Architecture

Assessment Tool 1 – Analytical Problem Solving (CO2)

# Question 1: 2's Complement Addition & Subtraction (+45 and −23)

## Step 1: Convert to 8-bit binary

- +45 → 00101101

- 23 in binary = 00010111 → −23 in 2's complement = invert bits (11101000) + 1 = 11101001

## Step 2: Addition — (+45) + (−23)

00101101   (+45)

+ 11101001   (−23)

-----------

100010110   (9-bit raw sum)

Discard the carry-out of the MSB (standard rule in 2's complement addition): result = 00010110 = 22 (decimal).

Check: 45 − 23 = 22 ✔

## Step 3: Overflow check for the addition

- Carry into sign bit (bit 7) = 1

- Carry out of sign bit (bit 7) = 1

Rule: overflow occurs only when carry-in to the sign bit ≠ carry-out of the sign bit. Here both are 1, so there is NO overflow. This makes sense — one operand is positive and one is negative, and the sum of a positive and a negative number can never overflow an n-bit signed representation.

## Step 4: Subtraction — (+45) − (−23)

Subtraction is implemented as addition of the 2's complement: (+45) − (−23) = (+45) + (+23).

00101101   (+45)

+ 00010111   (+23)

-----------

01000100   (68 decimal)

Check: 45 + 23 = 68 ✔

## Step 5: Overflow check for the subtraction

- Carry into sign bit = 0, Carry out of sign bit = 0 → equal → NO overflow

Also note both operands are positive and the result (sign bit = 0) is positive and within the representable range [−128, +127], confirming no overflow.

## Interpretation

Both operations produced correct, valid 8-bit signed results with no overflow, because overflow in 2's complement addition/subtraction can only occur when adding two numbers of the SAME sign yields a result of the opposite sign — which did not happen in either case here.

# Question 2: 4-bit Carry Look-Ahead Adder (CLA)

Example inputs: A = 1100 (12), B = 0110 (6), Cin = 0. (Bits numbered A3A2A1A0 / B3B2B1B0, bit 0 = LSB.)

## Step 1: Generate (Gi) and Propagate (Pi) signals

Gi = Ai · Bi  (a carry is generated at bit i)     Pi = Ai ⊕ Bi  (a carry is propagated through bit i)

| Bit i | Ai | Bi | Gi = Ai·Bi | Pi = Ai⊕Bi |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 | 1 |
| 2 | 1 | 1 | 1 | 0 |
| 3 | 1 | 0 | 0 | 1 |

## Step 2: Compute carries in advance using the CLA equations

- C1 = G0 + P0·C0 = 0 + 0·0 = 0

- C2 = G1 + P1·C1 = 0 + 1·0 = 0

- C3 = G2 + P2·C2 = 1 + 0·0 = 1

- C4 (carry-out) = G3 + P3·C3 = 0 + 1·1 = 1

Every carry (C1–C4) is computed directly and simultaneously from G, P and C0 — none of them waits for the previous stage's sum to ripple through, which is the key idea of look-ahead.

## Step 3: Compute the sum bits

Si = Ai ⊕ Bi ⊕ Ci

- S0 = 0⊕0⊕0 = 0

- S1 = 0⊕1⊕0 = 1

- S2 = 1⊕1⊕0 = 0

- S3 = 1⊕0⊕1 = 0

Result = C4 S3S2S1S0 = 1 0010 = 18 (decimal). Check: 12 + 6 = 18 ✔

## Step 4: Delay comparison — CLA vs Ripple Carry Adder (RCA)

| Parameter | Ripple Carry Adder | Carry Look-Ahead Adder |
| --- | --- | --- |
| Carry dependency | Each stage waits for carry from previous stage | All carries computed in parallel from G, P, C0 |
| Delay (4-bit) | ≈ 2n gate delays (grows with n) | ≈ constant (2–3 gate delays) + 2 for sum |
| Hardware cost | Low | Higher (extra AND/OR gates) |
| Scalability | Delay grows linearly with word length | Delay grows much more slowly (needs multi-level look-ahead for wide words) |

## Interpretation

Because every Ci is derived directly from G, P and C0 instead of the previous full adder's output, the worst-case path length no longer scales with the number of bits n the way it does for RCA. This is exactly why modern high-performance processors use CLA (often in a hierarchical form for wide ALUs) — it removes the ripple bottleneck at the cost of extra gates.

# Question 3: Booth's Algorithm (−6 × +5)

Using 4-bit 2's complement registers: Multiplicand M = −6 = 1010, −M = 0110. Multiplier Q = +5 = 0101, Q₋₁ = 0, Accumulator A = 0000. Number of cycles = 4 (word length).

## Booth's rule reminder

- Q0 Q₋1 = 10 → A = A − M

- Q0 Q₋1 = 01 → A = A + M

- Q0 Q₋1 = 00 or 11 → no arithmetic operation

- After every operation: arithmetic shift right the combined register A-Q-Q₋1 (sign bit of A is preserved)

## Step-by-step execution

| Cycle | Q0 Q₋1 | Operation | A (after op) | A-Q-Q₋1 after shift |
| --- | --- | --- | --- | --- |
| 1 | 1 0 | A = A − M | 0110 | A=0011 Q=0010 Q₋1=1 |
| 2 | 0 1 | A = A + M | 1101 | A=1110 Q=1001 Q₋1=0 |
| 3 | 1 0 | A = A − M | 0100 | A=0010 Q=0100 Q₋1=1 |
| 4 | 0 1 | A = A + M | 1100 | A=1110 Q=0010 Q₋1=0 |

The accumulator holds the running partial sum/difference, the multiplier register Q is shifted right one bit each cycle to bring in a new pair of bits, and Q₋1 remembers the bit just shifted out — together Q0Q₋1 tell Booth's algorithm whether a run of 1s is starting, continuing, or ending.

## Final result

After 4 cycles: A = 1110, Q = 0010 → Product = A Q = 1110 0010 (8-bit 2's complement).

Converting: 11100010 → invert (00011101) + 1 = 00011110 = 30 → so the value is −30.

Check: (−6) × (+5) = −30 ✔

## Interpretation

Booth's algorithm needed only two arithmetic operations (one subtraction, one addition) instead of examining every '1' bit individually as in simple shift-add multiplication. By encoding a run of consecutive 1s as a single subtract-then-add pair, it reduces the number of add/subtract operations, and because it works directly on 2's complement operands it handles signed multiplication (like −6 × +5) correctly without any special sign-correction step.

# Question 4: Division of 13 ÷ 3 — Restoring vs Non-Restoring

4-bit unsigned registers. Dividend Q = 1101 (13), Divisor M = 0011 (3), Accumulator A = 00000 (5-bit, to hold the sign during subtraction). n = 4 iterations.

## A. Restoring Division

Rule each cycle: (1) shift the combined A-Q register left by 1 bit. (2) A = A − M. (3) If A is negative (sign bit = 1): set Q0 = 0 and RESTORE by A = A + M. If A is non-negative: set Q0 = 1 (keep A).

| Cycle | After shift (A, Q) | A − M | Sign | Action | A, Q after cycle |
| --- | --- | --- | --- | --- | --- |
| 1 | 00001, 1010 | 11110 | − | Q0=0, restore A=A+M | 00001, 1010 |
| 2 | 00011, 0100 | 00000 | + | Q0=1, keep A | 00000, 0101 |
| 3 | 00000, 1010 | 11101 | − | Q0=0, restore A=A+M | 00000, 1010 |
| 4 | 00001, 0100 | 11110 | − | Q0=0, restore A=A+M | 00001, 0100 |

Final: Quotient Q = 0100 = 4, Remainder A = 00001 = 1. Check: 3 × 4 + 1 = 13 ✔

## B. Non-Restoring Division

Rule each cycle: if A is negative, shift A-Q left then A = A + M; if A is non-negative, shift left then A = A − M. Set Q0 = 1 if the new A is non-negative, else Q0 = 0. No immediate restore — only one correction at the very end if the final A is negative.

| Cycle | Prev. sign of A | Shift then op | A result | Sign | Q0 set |
| --- | --- | --- | --- | --- | --- |
| 1 | + (start) | shift, A = A − M | 11110 | − | 0 |
| 2 | − | shift, A = A + M | 00000 | + | 1 |
| 3 | + | shift, A = A − M | 11101 | − | 0 |
| 4 | − | shift, A = A + M | 11110 | − | 0 |

Final A is negative → one correction: A = A + M = 11110 + 00011 = 00001.

Final: Quotient Q = 0100 = 4, Remainder A = 00001 = 1. Check: 3 × 4 + 1 = 13 ✔ (same result as restoring division)

## Step: Comparison

| Aspect | Restoring Division | Non-Restoring Division |
| --- | --- | --- |
| Operations per cycle | Subtract, and possibly an extra restoring addition | Exactly one add OR subtract |
| Worst-case ops (4 bits) | Up to 8 (4 subtracts + up to 4 restores) | 4 (plus at most 1 final correction) = 5 |
| Hardware complexity | Simpler control, but wastes cycles restoring | Needs sign-tracking logic, but no wasted restore cycles |
| Speed | Slower (restore step adds delay) | Faster — preferred in modern processors |

## Interpretation

Both methods give the same correct quotient (4) and remainder (1). Non-restoring division is generally preferred in hardware because it never 'wastes' a cycle restoring the accumulator — it simply remembers the sign and picks add or subtract on the next cycle, needing at most one corrective addition at the end. This makes it faster and more efficient than restoring division for the same result.

# Question 5: IEEE 754 Representation & Addition (−13.25)

## Step 1: Convert −13.25 to binary

- 13 (integer part) = 1101

- 0.25 (fraction part) = 0.01

- So 13.25 = 1101.01₂, and the sign is negative

## Step 2: Normalize

1101.01 = 1.10101 × 2³  (moved the binary point 3 places left, so exponent = 3)

## Step 3: Single precision (32-bit) fields

- Sign (1 bit) = 1 (negative)

- Exponent (8 bits) = actual exponent + bias(127) = 3 + 127 = 130 = 10000010

- Mantissa/Fraction (23 bits) = the bits after the leading 1: 10101 followed by zero-padding = 10101000000000000000000

Single precision bit pattern:  1  10000010  10101000000000000000000

## Step 4: Double precision (64-bit) fields

- Sign (1 bit) = 1

- Exponent (11 bits) = 3 + bias(1023) = 1026 = 10000000010

- Mantissa/Fraction (52 bits) = 10101 followed by zero-padding to 52 bits

Double precision bit pattern: 1  10000000010  1010100000000000000000000000000000000000000000000000

## Step 5: Floating-point addition example (−13.25 + 5.75)

To illustrate FP addition, add −13.25 to 5.75 (5.75 = 101.11₂ = 1.0111 × 2²).

Step 1: Align exponents to the larger one (3): 1.0111 × 2² = 0.10111 × 2³.

Step 2: Since the signs differ, subtract magnitudes: 1.10101 − 0.10111 = 0.11110 (result stays negative, matching the larger-magnitude operand).

Step 3: Result so far: −0.11110 × 2³ — the leading bit is 0, so it is NOT normalized.

Step 4: Normalize: shift the mantissa left by 1 and decrease the exponent by 1 → −1.1110 × 2².

Step 5: Convert back to decimal: −1.1110 × 2² = −111.10₂ = −7.5

Check: −13.25 + 5.75 = −7.5 ✔

## Interpretation

This example required a post-subtraction normalization step (shifting the mantissa and adjusting the exponent) because subtracting two aligned mantissas can produce leading zeros. In general, IEEE-754 addition/subtraction can also require rounding when the result's mantissa needs more bits than the format allows (23 for single, 52 for double); the extra bits are rounded off using guard, round, and sticky bits, which is the source of precision loss / rounding error in floating-point arithmetic. Double precision keeps far more mantissa and exponent bits than single precision, so it represents the same value with much higher precision and a wider exponent range — important for scientific computing where accumulated rounding error must stay small.