SIMATS ENGINEERING

# Department of Computer Science and Engineering

ASSESSMENT TOOL 2 – DEBUGGING & OPTIMISATION TASK

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO2 – Precision (BL3) | Assessment: | Debugging and Optimisation task |
| Weightage: | 45% | Total Marks: | 100 |
| CO2 Statement: | Apply integer and floating-point arithmetic algorithms including Booth's multiplication, restoring/nonrestoring division, and IEEE 754 standard operations to solve fixed-point and floating-point computational problems. |  |  |
| Student Name: | J. LAKSHMI DEVI | Reg. No.: | 192524300 |
| Section / Batch: | 2025 | Date: | 05.08.2026 |

Instructions to Students - Identify arithmetic errors and performance bottlenecks in the given problem. - Analyse the execution steps to locate the source of errors. - Debug the algorithm by correcting logical and computational faults. - Optimize the solution using appropriate arithmetic techniques or algorithms. - Evaluate the optimized solution for correctness, accuracy, and efficiency.

# QUESTIONS

- A program performing signed integer addition using 2’s complement produces incorrect results when adding large values (e.g., +120 and +50 in 8-bit representation). Debug the issue by analyzing the binary computation and identifying whether overflow has occurred. Propose a solution to correctly detect and handle overflow in such cases.

- A processor using a ripple carry adder shows significant delay in executing arithmetic operations in a realtime system. Debug the performance bottleneck by examining how carry propagates through each bit. Suggest an optimized solution using a carry look-ahead adder, and explain how it reduces delay.

- A multiplication unit implementing Booth’s algorithm gives incorrect results for certain negative operands. Debug the step-by-step execution of the algorithm, focusing on bit-pair checking, arithmetic shifts, and sign extension. Identify the possible source of error and suggest corrections to ensure accurate signed multiplication.

- An embedded system using the restoring division algorithm experiences inefficiency due to repeated restoration steps, leading to increased execution time. Debug the algorithm’s workflow and identify where unnecessary operations occur. Propose an optimized approach using the non-restoring division method, explaining how it improves performance.

- A scientific application using IEEE 754 single precision floating-point representation produces inaccurate results when adding very small and very large numbers. Debug the issue by analyzing exponent alignment, normalization, and rounding errors. Suggest optimization techniques, such as using double precision or algorithmic adjustments, to improve accuracy.

# Code of Conduct

I J. LAKSHMI DEVI (192524300) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action.

# RUBRICS FOR CALCULATION

| Criteria | Weightage(%) | Level 4 – Exemplary (5 Marks) | Level 3 – Proficient (4 Marks) | Level 2 – Developing (2–3 Marks) | Level 1 – Beginning (0–1 Mark) |
| --- | --- | --- | --- | --- | --- |
| Identification of Errors / Bugs | 20 | Accurately identifies all errors and issues with complete understanding of the problem | Identifies most errors with minor omissions | Identifies limited errors; misses key issues | Unable to identify errors or misunderstands the problem |
| Debugging Approach & Analysis | 20 | Uses effective debugging techniques with clear analysis and root cause identification | Applies suitable debugging methods with minor gaps | Uses basic debugging methods with limited analysis | No systematic debugging approach followed |
| Correctness of Debugged Solution | 25 | Provides completely corrected solution with accurate functionality and expected output | Provides working solution with minor errors | Partially correct solution with limited functionality | Incorrect solution or fails to resolve errors |
| Optimization Techniques Applied | 20 | Applies efficient optimization methods to improve performance, memory usage and quality | Performs optimization with noticeable improvements | Limited optimization with minimal improvements | No optimization applied or inefficient implementation |
| Testing and Documentation | 15 | Performs complete testing with proper validation and clear documentation | Provides adequate testing and documentation with minor issues | Limited testing and incomplete documentation | No proper testing or documentation provided |

Marks Evaluation:

| Q. No | Identification of Errors / Bugs (20) |  | Debugging Approach & Analysis (20) | Correctness of Debugged Solution (25) | Optimization Techniques Applied (20) | Testing and Documentation (15) | Total (out of 100) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 2 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 3 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 4 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 5 | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
|  | / 20 |  | / 20 | / 25 | / 20 | / 15 | / 100 |

# Course Coordinator  	 	              	 	 	Faculty Incharge

CSA1220 – COMPUTER ARCHITECTURE

CO2 Assessment Tool 2 – Debugging & Optimisation

# Question 1: Overflow Bug in 8-bit Signed Addition (+120 and +50)

Step 1: Reproduce the computation

+120 = 01111000        +50 = 00110010

0 1 1 1 1 0 0 0

+  0 0 1 1 0 0 1 0

---------------------

1 0 1 0 1 0 1 0

The raw 8-bit result is 10101010. Decoded as a signed 2's-complement value: invert → 01010101, add 1 → 01010110 = 86, so the program reports −86.

Step 2: Analyse — locate the source of the error

Mathematically 120 + 50 = 170, which cannot be represented in an 8-bit signed field (valid range is −128 to +127). The hardware still produces a bit pattern (10101010), but because the sign bit (bit 7) flipped to 1, the program silently reads it as a negative number (−86) instead of recognizing that the addition is invalid in this width.

Trace the carries into and out of the sign bit (bit 7) to confirm this is a genuine overflow, not a normal wraparound:

| Bit position | 6 | 7 (sign) |
| --- | --- | --- |
| Carry generated here | 1 (into bit 7) | 0 (out of bit 7) |

Bug Identified: The program adds the two operands and directly trusts the sign bit of the result without checking the overflow condition. Carry-into-MSB (1) XOR Carry-out-of-MSB (0) = 1, which signals overflow — two positive operands have incorrectly produced a negative result (−86 instead of the true sum 170). The root cause is a missing overflow check, not a fault in the adder itself.

Step 3: Debugged solution — correct overflow detection

Add an explicit overflow-detection condition immediately after every signed addition, using either of these equivalent hardware-friendly rules:

- Carry-based rule: Overflow = Carry-into-MSB XOR Carry-out-of-MSB.

- Sign-based rule: Overflow occurs only when both operands have the SAME sign and the result's sign differs from theirs (i.e., (Sign A = Sign B) AND (Sign Result ≠ Sign A)).

Applying the sign-based rule here: Sign(+120)=0, Sign(+50)=0 (same), Sign(result)=1 (different) → Overflow = TRUE, confirming the bug programmatically.

Step 4: Optimized handling once overflow is detected

- Set a hardware Overflow (OV) flag and raise an exception/trap so the program never silently trusts the corrupted value.

- Where valid, saturate the result to the maximum representable value (+127) instead of wrapping to a negative number, for applications like signal/image processing.

- If the true numeric value must be preserved, promote the operands to a wider register (e.g., 16-bit) before adding: 120 + 50 = 170 = 0000000010101010₂, which is exactly representable in 16-bit signed.

Step 5: Testing / verification

Re-running the corrected routine with the overflow check on (+120, +50): the OV flag is now raised, so the routine reports "overflow — result not valid in 8-bit" instead of returning −86. Using 16-bit operands instead, 0000000001111000 + 0000000000110010 = 0000000010101010 = +170, verified correct with no overflow (OV = 0, since 16-bit range covers up to 32767).

# Question 2: Ripple Carry Adder Delay Bottleneck

Step 1: Reproduce the bottleneck

Worst-case example: A = 11111111 (255), B = 00000001 (1). In a Ripple Carry Adder (RCA), each bit's Full Adder cannot compute its sum/carry until the carry from the previous (less significant) bit arrives.

| Bit | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carry-out generated at | t=1Δ | t=2Δ | t=3Δ | t=4Δ | t=5Δ | t=6Δ | t=7Δ | t=8Δ (final) |

Step 2: Analyse — locate the source of the delay

Bug Identified: Every Full Adder stage must wait for the carry-out of the stage before it — this is a strictly sequential dependency chain. For an n-bit RCA the worst-case delay grows linearly, O(n) × (delay of one Full Adder), because bit 7 cannot settle until bits 0–6 have all resolved their carries in sequence. In a real-time system this cumulative latency directly limits the maximum clock frequency.

Step 3: Optimized solution — Carry Look-Ahead Adder (CLA)

Define, for each bit i, Generate Gi = Ai·Bi (produces a carry on its own) and Propagate Pi = Ai⊕Bi (passes an incoming carry through). Every carry is then expressed directly from the primary inputs and C0 using two-level AND–OR logic, so no stage has to wait for its neighbour:

C1 = G0 + P0C0

C2 = G1 + P1G0 + P1P0C0

C3 = G2 + P2G1 + P2P1G0 + P2P1P0C0

...  Cn is generated the same way, purely from Gi, Pi, and C0

Step 4: Verify with the same operands

For A = 11111111, B = 00000001: Gi = Ai·Bi is 0 everywhere except bit 0 (G0 = 1); Pi = Ai⊕Bi is 1 for bits 1–7 and 0 for bit 0. Substituting into the look-ahead equations, C1 = G0 = 1, and every subsequent carry Ck = Pk-1Pk-2...P0·C1... = 1 (since all the propagate terms above bit 0 are 1), so every carry resolves the moment G0/C0 are known — in roughly one AND-OR gate delay per 4-bit block, instead of rippling through all 8 stages.

Step 5: Performance comparison

| Aspect | Ripple Carry Adder | Carry Look-Ahead Adder |
| --- | --- | --- |
| Carry dependency | Sequential, stage-by-stage | Parallel, direct from G/P/C0 |
| Delay for n bits | O(n) | O(log n) (near-constant per block) |
| Hardware cost | Low | Higher (extra AND-OR network) |
| Suitability | Poor for real-time / high-speed paths | Well suited to real-time, high-frequency ALUs |

# uestion 3: Booth's Algorithm Error with Negative Operands (+9 × −6)

Step 1: Set up the correct execution

M (multiplicand) = +9 = 01001,    −M = 10111

Q (multiplier)   = −6 = 11010

Initialize A = 00000, Q = 11010, Q-1 = 0. The rule: Q0Q-1 = 10 → A=A−M; 01 → A=A+M; 00 or 11 → no operation; every cycle ends with an ARITHMETIC right shift (sign bit of A replicated into the new MSB, not filled with 0).

Step 2: Trace where the bug appears

A faulty implementation sometimes uses a LOGICAL right shift (always inserting 0 at the top) instead of an ARITHMETIC right shift. The two trace identically until A first becomes negative — at iteration 2 here:

| Iter | Q0 Q-1 | Op | A before shift | Correct (arithmetic) shift → A | Faulty (logical) shift → A |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 0 | No operation | 00000 | 00000 | 00000 |
| 2 | 1 0 | A = A − M → 10111 | 10111 | 11011 (sign bit 1 replicated) | 01011 (WRONG — 0 inserted, sign lost) |

Bug Identified: Using a logical shift instead of an arithmetic shift on a negative accumulator destroys the sign bit (A becomes 01011, positive, instead of the correct 11011, negative). From this point every remaining Booth decision and every subsequent shift operates on a corrupted, wrong-signed accumulator, so the final product comes out incorrect whenever the accumulator goes negative during the algorithm — which happens precisely when operands are negative. A second common variant of the same bug is computing −M as a plain bit-inversion (one's complement) without adding 1, which also silently corrupts every subtraction step.

Step 3: Corrected execution (arithmetic shift throughout)

| Iter | Q0 Q-1 | Operation | A (after op → after shift) | Q (after shift) | Q-1 |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 0 | No operation | 00000 → 00000 | 01101 | 0 |
| 2 | 1 0 | A = A − M | 10111 → 11011 | 10110 | 1 |
| 3 | 0 1 | A = A + M | 00100 → 00010 | 01011 | 0 |
| 4 | 1 0 | A = A − M | 11001 → 11100 | 10101 | 1 |
| 5 | 1 1 | No operation | 11100 → 11110 | 01010 | 1 |

Step 4: Verify the corrected result

Final A = 11110, Q = 01010, so product = A‖Q = 1111001010 (10 bits). Decoding: invert → 0000110101, add 1 → 0000110110 = 54, sign bit 1 → value = −54.

Check: (+9) × (−6) = −54 ✔ — matches, confirming the arithmetic-shift version is correct.

Step 5: Optimization / prevention

- Always implement the post-operation shift as an arithmetic right shift (replicate the current sign bit of A), never a logical shift.

- Always compute −M as a true 2's complement (invert all bits, then add 1) rather than a simple bit-flip.

- Add a unit test with at least one operand negative (as done above) — bugs that only appear when the accumulator goes negative will not be caught by all-positive test cases.

# Question 4: Restoring Division Inefficiency (19 ÷ 5)

Step 1: Reproduce with Restoring Division

Setup (5-bit registers): Dividend = 19 = 10011 loaded into Q, Divisor M = 5 = 00101, A = 00000, n = 5 iterations. Rule: shift A,Q left; A = A − M; if A < 0, RESTORE (A = A + M) and set Q0 = 0; else set Q0 = 1.

| Iter | After shift (A,Q) | A − M | Sign | Action taken | Final A | Final Q |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 00001, 00110 | 11100 | −ve | Restore (extra add), Q0=0 | 00001 | 00110 |
| 2 | 00010, 01100 | 11101 | −ve | Restore (extra add), Q0=0 | 00010 | 01100 |
| 3 | 00100, 11000 | 11111 | −ve | Restore (extra add), Q0=0 | 00100 | 11000 |
| 4 | 01001, 10000 | 00100 | +ve | Keep, Q0=1 | 00100 | 10001 |
| 5 | 01001, 00010 | 00100 | +ve | Keep, Q0=1 | 00100 | 00011 |

Final: Quotient Q = 00011 = 3, Remainder A = 00100 = 4 (Check: 5×3=15, 19−15=4 ✔).

Bug Identified: In 3 of the 5 iterations (60% of the algorithm's steps) the subtraction A−M produces a negative result that must immediately be undone with an extra restoring addition. Every restore is wasted work: the subtract, the sign test, AND the restoring add are all executed just to end up back where the accumulator started. Counting operations: 5 shifts + 5 subtracts + 3 restoring adds = 8 arithmetic operations for this one division — the 3 restores contribute nothing to the final answer, only to detecting the sign.

Step 2: Optimized solution — Non-Restoring Division

Rule: if A ≥ 0, shift then A = A − M; if A < 0, shift then A = A + M (no separate restore step — the sign of A is simply carried forward and corrected on the NEXT iteration's choice of add/subtract). Set Q0 = 1 if the result is non-negative, else Q0 = 0. Only after the very last iteration, if A is still negative, one single final correction (A = A + M) is applied.

| Iter | A before | Shift → (A,Q) | Operation | A after op | Sign | Q0 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 00000 (+ve) | 00001, 00110 | A − M | 11100 | −ve | 0 |
| 2 | 11100 (−ve) | 11000, 01100 | A + M | 11101 | −ve | 0 |
| 3 | 11101 (−ve) | 11010, 11000 | A + M | 11111 | −ve | 0 |
| 4 | 11111 (−ve) | 11111, 10000 | A + M | 00100 | +ve | 1 |
| 5 | 00100 (+ve) | 01001, 10001 | A − M | 00100 | +ve | 1 |

A ends non-negative (00100), so no final correction is needed. Quotient Q = 00011 = 3, Remainder A = 00100 = 4 — identical to the restoring-division result, confirming correctness.

Step 3: Quantify the optimization

| Method | Shifts | Add/Subtract ops | Restore ops | Total arithmetic ops |
| --- | --- | --- | --- | --- |
| Restoring Division | 5 | 5 | 3 | 13 |
| Non-Restoring Division | 5 | 5 | 0 (max 1 final correction) | 10 (worst case) |

# Question 5: IEEE 754 Precision Loss Adding Very Large and Very Small Numbers

Step 1: Reproduce the issue with a concrete example

Add A = 1.0 and B = 2⁻²⁵ (a very small number) in IEEE 754 single precision.

A = 1.0 = 1.00000000000000000000000 × 2^0   (exponent = 0)

B = 2^-25 = 1.00000000000000000000000 × 2^-25  (exponent = −25)

Step 2: Exponent alignment (this is where the bug appears)

To add A and B, B's mantissa must be shifted right by (0 − (−25)) = 25 bit positions to match A's exponent. Single precision has only 23 fraction bits (24 significant bits including the implicit leading 1).

Bug Identified: Shifting B's mantissa right by 25 bits pushes its entire significant '1' bit beyond the 23-bit fraction field — there is no room left to store any part of B once aligned. After rounding, B's contribution rounds to exactly 0, so the floating-point unit computes A + B = 1.0, silently discarding B entirely, even though the mathematically exact sum is 1.0000000298023223... This is precisely the reported symptom: adding a very small number to a very large one produces an inaccurate (unchanged) result.

Step 3: Confirm this is expected IEEE 754 single-precision behaviour

The smallest gap between representable single-precision numbers near 1.0 (the machine epsilon) is 2⁻²³ ≈ 1.19 × 10⁻⁷. Since 2⁻²⁵ is only 1/4 of that gap, it is smaller than half the rounding interval, so it rounds away to nothing — this is a genuine precision limitation of the 23-bit mantissa, not a coding mistake in the adder itself.

Step 4: Optimized fix #1 — use double precision

Double precision carries 52 fraction bits, so the gap near 1.0 shrinks to 2⁻⁵² ≈ 2.22 × 10⁻¹⁶. Repeating the same addition in double precision: B = 2⁻²⁵ needs only a 25-bit right shift, which comfortably fits inside the 52-bit mantissa, so B's value is retained and A + B = 1.0000000298023223876953125 is computed correctly (exactly, in this case, since 2⁻²⁵ is a power of two).

Step 5: Optimized fix #2 — algorithmic adjustments

- Reorder summation so that small-magnitude terms are added together first before being combined with large terms (pairwise summation), reducing the chance that any single addition has a huge exponent gap.

- Use Kahan (compensated) summation, which keeps a running "lost low-order bits" correction term and adds it back in on subsequent additions, recovering much of the precision that plain repeated addition would discard.

- Avoid unnecessary mixing of very different magnitudes in one expression where possible — e.g., accumulate small quantities separately and merge with the large quantity only once, at the end.

Step 6: Testing / verification

Single precision test: 1.0 + 2⁻²⁵ → result bit pattern identical to 1.0 (bug reproduced). Double precision test: 1.0 + 2⁻²⁵ → result differs from 1.0 in the low mantissa bits, correctly reflecting the added quantity (bug resolved).