SIMATS ENGINEERING

Department of Computer Science and Engineering

# ASSESSMENT TOOL 1 – INDUSTRY PROBLEM SOLVING TASK

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO4 – Precision (BL4) | Assessment: | Industry Problem solving task |
| Weightage: | 30% | Total Marks: | 100 |
| CO4 Statement: | Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics. |  |  |
| Student Name: | LAKSHMI DEVI.J | Reg. No.: | 192524300 |
| Section / Batch: | 2025 | Date: | 17.08.2026 |

Instructions to Students - Identify the relevant concepts of cache hierarchy, SRAM, DRAM, MRAM, NAND Flash, RRAM, memory latency, bandwidth, and virtual memory paging. - Analyze the memory-access requirements of smartphones, cloud servers, autonomous vehicles, edge AI devices, and gaming consoles, considering latency, bandwidth, capacity, and power consumption. - Apply suitable memory-hierarchy techniques such as cache optimization, appropriate memory technology selection, data locality, paging optimization, and hierarchical memory organization. - Compute performance measures such as memory access time, latency, bandwidth, throughput, speedup, hit/miss rates, and resource or power utilization where applicable. - Interpret the results by evaluating performance improvements, memory bottlenecks, technology trade-offs, and the suitability of the proposed memory hierarchy for each application.

QUESTIONS

- A mobile device manufacturer observes noticeable lag when users switch rapidly between multiple applications. Analyze how L1, L2, and L3 caches and DRAM participate in storing and retrieving frequently accessed application data. Based on latency, capacity, hit rate, and bandwidth requirements, propose a suitable hierarchical memory organization that can reduce application switching delays and improve overall system throughput.

1. Mobile Device – Application Switching Delay

Problem:
Rapid switching between applications causes noticeable lag because the processor frequently needs to retrieve application instructions and data from lower levels of memory.

Analysis:

- L1 Cache: Very small but extremely fast. Stores the most frequently accessed instructions and data close to the CPU.

- L2 Cache: Larger than L1 with slightly higher latency. Stores data that cannot fit in L1.

- L3 Cache: Much larger and usually shared among CPU cores. Helps reduce accesses to DRAM.

- DRAM: Provides high capacity for application data but has significantly higher latency than cache memories.

Proposed Memory Hierarchy:

CPU → L1 Cache → L2 Cache → Shared L3 Cache → DRAM → Storage

- Use a large, low-latency L1 cache for active application instructions.

- Use larger L2 caches to retain recently used application data.

- Use a shared L3 cache to reduce DRAM accesses when switching applications.

- Use high-bandwidth LPDDR5/LPDDR5X DRAM as the main memory.

- Apply intelligent cache replacement and prefetching to predict application data requirements.

Expected Result:
Higher cache hit rates reduce expensive DRAM accesses, decreasing application-switching latency and improving overall CPU throughput.

- A cloud service provider experiences increased latency when processing database queries from a large number of concurrent users. Analyze how cache hierarchy and virtual memory paging influence memory access performance, and compare their roles in reducing processor-memory latency. Recommend suitable memory technologies and memory-management improvements that can minimize page faults, reduce access delays, and improve database throughput.

2. Cloud Database – High Latency with Concurrent Users

Problem:
A cloud database serving many users generates a large number of memory accesses. Cache misses and virtual-memory page faults can significantly increase query-processing time.

Analysis:

- Cache hierarchy reduces processor-memory latency by keeping frequently accessed database instructions and data close to the CPU.

- L1 cache provides the lowest latency but has limited capacity.

- L2/L3 caches provide larger storage for frequently reused database data.

- DRAM stores the main working set of database processes.

- Virtual memory allows applications to use more address space than available physical RAM.

- When required pages are not in RAM, a page fault occurs and data must be obtained from secondary storage, causing a major delay.

Comparison:

| Feature | Cache Hierarchy | Virtual Memory |
| --- | --- | --- |
| Main purpose | Reduce CPU-memory latency | Extend available memory |
| Speed | Very high | Slower during page faults |
| Storage | Very small | Much larger |
| Main benefit | Faster repeated accesses | Supports large workloads |
| Performance issue | Cache misses | Page faults |

Recommended Improvements:

- Increase available high-speed DRAM to keep the database working set in memory.

- Use large L3 caches and cache-friendly database structures.

- Use NVMe SSDs instead of slower storage for paging when paging cannot be avoided.

- Reduce unnecessary paging through better memory allocation.

- Use huge pages where appropriate to reduce page-table overhead.

- Apply database buffer-pool optimization and frequently accessed-data caching.

- Monitor memory utilization and page-fault rates.

Expected Result:
Higher cache hit rates and fewer page faults reduce memory-access delays and improve database query throughput for concurrent users.

- An autonomous vehicle continuously receives large volumes of sensor data from cameras, LiDAR, radar, and other real-time sensing systems. Evaluate the characteristics of SRAM, DRAM, and MRAM in terms of access latency, bandwidth, capacity, volatility, and power consumption. Based on the real-time processing requirements, design an appropriate memory hierarchy that can provide high data-transfer bandwidth while minimizing access latency and supporting reliable sensor-data processing.

3. Autonomous Vehicle – Real-Time Sensor Processing

Problem:
Autonomous vehicles continuously process large data streams from cameras, LiDAR, radar, and other sensors. The memory system must provide high bandwidth with very low latency.

Memory Comparison

| Characteristic | SRAM | DRAM | MRAM |
| --- | --- | --- | --- |
| Latency | Very low | Moderate | Low |
| Bandwidth | Very high | High | Moderate–High |
| Capacity | Low | High | Medium |
| Volatility | Volatile | Volatile | Non-volatile |
| Power | Higher per bit, but efficient for small caches | Moderate | Low standby power |
| Main use | CPU/cache/buffers | Main memory | Fast persistent storage/cache |

Proposed Hierarchy:

CPU/AI Accelerator → SRAM Cache → High-Bandwidth DRAM → MRAM → Persistent Storage

- Use SRAM for real-time sensor buffers and frequently accessed AI data.

- Use high-bandwidth DRAM for large camera, LiDAR, and radar datasets.

- Use MRAM for selected persistent or frequently accessed data where non-volatility is useful.

- Use fast storage for long-term sensor recordings.

Expected Result:
SRAM handles latency-sensitive operations, while DRAM provides the capacity and bandwidth needed for large sensor streams. This combination minimizes processing delays while maintaining reliable real-time operation.

- An edge-based AI device must load machine-learning models quickly while operating under strict power and storage constraints. Analyze the characteristics of NAND Flash, DRAM, and RRAM with respect to access speed, storage capacity, persistence, power consumption, and suitability for AI workloads. Recommend a hierarchical memory organization that enables fast model loading, efficient intermediate-data processing, reduced energy consumption, and improved overall inference performance.

4. Edge AI Device – Fast Machine-Learning Model Loading

Problem:
An edge AI device needs to load ML models quickly while operating with limited power, memory, and storage capacity.

Memory Comparison

| Characteristic | NAND Flash | DRAM | RRAM |
| --- | --- | --- | --- |
| Speed | Moderate | Very high | High |
| Capacity | Very high | Medium | Medium |
| Persistence | Non-volatile | Volatile | Non-volatile |
| Power | Low standby | Higher during operation | Low |
| AI suitability | Model storage | Intermediate processing | Fast/persistent AI memory |

Proposed Hierarchy:

AI Accelerator → DRAM → RRAM → NAND Flash

- Store large ML models permanently in NAND Flash.

- Copy frequently used models or model portions into RRAM for faster access and lower standby power.

- Use DRAM for temporary tensors, activations, intermediate calculations, and runtime workloads.

- Use model compression and quantization to reduce model size and memory traffic.

Expected Result:
NAND Flash provides high-capacity persistent storage, RRAM enables faster access to important model data, and DRAM handles high-speed intermediate processing. This reduces model-loading time and energy consumption while improving inference performance.

- A high-performance gaming console experiences frame drops and noticeable delays when loading large game environments containing high-resolution textures, audio, and other assets. Analyze the interaction between L3 cache and DRAM during the retrieval and processing of frequently accessed game data, considering their latency, capacity, and bandwidth. Propose suitable memory hierarchy enhancements that can reduce data-access bottlenecks, increase memory throughput, and provide smoother frame rendering during large scene transitions.

5. Gaming Console – Frame Drops During Large Scene Loading

Problem:
Large game environments contain high-resolution textures, audio, geometry, animations, and other assets. Frequent movement between scenes can generate substantial memory traffic and cause frame drops.

Analysis:

- L3 cache stores frequently reused game instructions and data close to CPU cores.

- It has much lower latency than DRAM but significantly smaller capacity.

- DRAM stores large game assets and active scene data.

- When required data is not available in the cache, the processor accesses DRAM, increasing latency.

- High-resolution textures and other large assets can generate substantial DRAM bandwidth requirements.

Proposed Hierarchy:

CPU/GPU → L1/L2 Cache → Large Shared L3 Cache → High-Bandwidth GDDR6/GDDR6X → NVMe SSD

- Increase L3 cache capacity to retain frequently reused game data.

- Use high-bandwidth GDDR memory for GPU-intensive texture and graphics workloads.

- Apply hardware/software prefetching to load assets before they are required.

- Stream game assets from fast NVMe SSD storage.

- Use texture compression to reduce memory bandwidth requirements.

- Keep frequently accessed textures and game objects in memory to reduce repeated storage accesses.

Expected Result:
A larger and efficient L3 cache reduces DRAM accesses, while high-bandwidth GDDR memory supplies the GPU with data quickly. Asset prefetching and SSD streaming reduce scene-transition delays, resulting in higher frame rates, fewer frame drops, and smoother rendering.

## Code of Conduct

I J. LAKSHMI DEVI (192524300) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action.

RUBRICS FOR CALCULATION

| Criteria | Weightage(%) | Level 4 – Exemplary (5 Marks) | Level 3 – Proficient (4 Marks) | Level 2 – Developing (2–3 Marks) | Level 1 – Beginning (0–1 Mark) |
| --- | --- | --- | --- | --- | --- |
| Understanding of Industry Problem | 20% | Comprehensive understanding of real-world problem and constraints | Good understanding with minor gaps in requirements | Basic understanding; some requirements unclear | Poor understanding; misinterprets problem context |
| Application of Engineering Concepts | 25% | Applies advanced and relevant concepts effectively to solve the problem | Applies appropriate concepts with minor errors | Limited application of concepts | Incorrect or no application of concepts |
| Solution Design | 25% | Innovative, practical, and feasible solution aligned with industry standards | Logical and feasible solution with minor gaps | Basic solution with limited feasibility | Unclear or impractical solution |
| Use of Tools | 15% | Effective use of modern tools, technologies, and real data | Appropriate use of tools with correct implementation | Limited or inconsistent use of tools | Minimal or incorrect use of tools |
| Analysis | 10% | Thorough analysis with validated results and performance evaluation | Good analysis with reasonable validation | Basic analysis with limited validation | Poor or no analysis |
| Professionalism | 5% | Highly professional, well-documented, and clearly presented work | Good documentation and presentation | Basic documentation with some gaps | Poor documentation and presentation |

Marks Evaluation:

| Q. No | Understanding of Industry Problem (20) |  | Application of Engineering Concepts (25) |  | Solution Design (25) | Use of Tools (15) | Analysis (10) | Professiona lism(5) | Total (out of 100) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 2 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 3 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 4 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
| 5 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 | / 4 |  |
|  | / 25 |  | / 30 |  | / 20 | / 15 | / 10 | /5 | / 100 |

Course Coordinator  	 	              	 	 	Faculty Incharge