SIMATS ENGINEERING

Department of Computer Science and Engineering

# ASSESSMENT TOOL 2 – COMPARATIVE ANALYSIS

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO4 – Precision (BL4) | Assessment: | Comparative Analysis |
| Weightage: | 30% | Total Marks: | 100 |
| CO4 Statement: | Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics. |  |  |
| Student Name: | LAKSHMI DEVI J | Reg. No.: | 192524300 |
| Section / Batch: | 2025 | Date: | 24.08.26 |

Instructions to Students - Identify the relevant concepts of L1, L2, and L3 cache, SRAM, DRAM, NAND Flash, MRAM, RRAM, virtual memory, paging, latency, bandwidth, and throughput. - Analyze the memory requirements of different computer systems by considering cache levels, memory technologies, access latency, bandwidth, throughput, capacity, and energy consumption. - Evaluate the performance characteristics of different memory technologies and determine their suitability for cache memory, main memory, secondary storage, and future hierarchical memory systems. - Compute relevant performance measures such as access time, latency, bandwidth, throughput, hit/miss rate, average memory access time (AMAT), speedup, and energy efficiency wherever applicable. - Interpret the results by assessing memory bottlenecks, performance improvements, technology trade-offs, and the suitability of each memory technology or memory hierarchy for a given application.

QUESTIONS

- Evaluate L1, L2, and L3 cache levels based on access latency, bandwidth, and throughput. Analyze how each cache level influences processor performance and memory access efficiency.

## 1. Evaluation of L1, L2, and L3 Cache

| Cache Level | Access Latency | Bandwidth | Throughput | Effect on Performance |
| --- | --- | --- | --- | --- |
| L1 Cache | Very low, typically ~1–4 cycles | Very high | Very high | Provides data/instructions to CPU extremely quickly |
| L2 Cache | Low, typically ~5–20 cycles | High | High | Reduces accesses to slower L3 cache and RAM |
| L3 Cache | Higher, typically ~20–50+ cycles | Lower than L1/L2 | Moderate to high | Reduces expensive main-memory accesses and is often shared among CPU cores |

Analysis:
L1 is the smallest and fastest cache and is usually located closest to the CPU core. It stores frequently used instructions and data. L2 is larger but slightly slower and acts as a secondary cache when data is not found in L1. L3 is larger still and is commonly shared by multiple cores. Although slower than L1 and L2, it significantly reduces the number of accesses to DRAM.

The cache hierarchy improves memory access efficiency through temporal and spatial locality. A cache hit allows the processor to obtain data quickly, whereas a cache miss causes access to a slower level. Therefore, a high cache-hit rate improves processor throughput, reduces CPU waiting time, and increases overall system performance.

- Assess the performance characteristics of SRAM and DRAM with respect to access latency, bandwidth, and throughput. Justify the use of SRAM in cache memory and DRAM in main memory based on their respective characteristics.

## 2. SRAM vs DRAM

| Characteristic | SRAM | DRAM |
| --- | --- | --- |
| Access Latency | Very low | Higher |
| Bandwidth | Very high | High |
| Throughput | Very high | High |
| Storage Density | Low | High |
| Cost per Bit | High | Low |
| Refresh Required | No | Yes |
| Typical Application | CPU caches | Main memory |

Analysis:
SRAM stores each bit using a flip-flop-based circuit and does not require periodic refreshing. This makes it extremely fast and suitable for applications requiring very low access latency. However, SRAM requires more transistors per bit, making it expensive and less dense.

DRAM stores data using capacitors and therefore requires periodic refresh operations. It has higher latency than SRAM but provides much greater storage density at a lower cost per bit.

Justification:
SRAM is used for L1, L2, and L3 cache because cache memory must respond to the processor with very low latency and high throughput. DRAM is used as main memory because computers require large amounts of memory, and DRAM provides high capacity at a reasonable cost. Thus, SRAM prioritizes speed, while DRAM provides a better balance of capacity, cost, and performance.

- Examine virtual memory, paging, and cache memory mechanisms in terms of access latency, memory utilization, and system throughput. Analyze how these mechanisms contribute to efficient memory management and overall system performance.

## 3. Virtual Memory, Paging, and Cache Memory

| Mechanism | Access Latency | Memory Utilization | Effect on Throughput |
| --- | --- | --- | --- |
| Cache Memory | Very low | Stores frequently used data | Increases CPU throughput |
| Paging | Moderate; page-table/TLB access adds overhead | Efficiently allocates physical memory | Supports multitasking but page faults reduce throughput |
| Virtual Memory | Higher when disk/SSD access is required | Allows programs to use an address space larger than physical RAM | Improves system flexibility but excessive swapping reduces performance |

Cache memory keeps frequently accessed data close to the CPU. When a cache hit occurs, the processor avoids slower main-memory access. This greatly reduces effective memory-access latency.

Paging divides virtual and physical memory into fixed-size pages and frames. It allows processes to use memory efficiently without requiring their entire address space to be physically contiguous. The operating system can move pages between RAM and secondary storage when required.

Virtual memory provides each process with a large logical address space independent of the actual physical RAM available. It improves memory utilization and allows more programs to execute concurrently. However, a page fault requires data to be retrieved from secondary storage, which is much slower than accessing RAM.

Overall, these mechanisms work together: cache reduces CPU-to-memory latency, paging manages physical memory efficiently, and virtual memory provides a large logical memory space. Proper use of all three improves system performance and multitasking capability.

- Analyze the performance characteristics of NAND Flash and DRAM with respect to access latency, bandwidth, and throughput. Evaluate their suitability for different levels of a hierarchical memory system and justify their applications.

## 4. NAND Flash vs DRAM

| Characteristic | DRAM | NAND Flash |
| --- | --- | --- |
| Access Latency | Very low, typically tens of nanoseconds | Much higher, especially for writes |
| Bandwidth | Very high | High, especially in modern SSDs |
| Throughput | Very high for frequent random access | High for storage workloads, particularly sequential access |
| Volatility | Volatile | Non-volatile |
| Endurance | Extremely high for normal memory operations | Limited program/erase cycles |
| Main Application | Main memory | SSDs and persistent storage |

Analysis:
DRAM is much faster than NAND Flash for random memory access. It is therefore positioned close to the processor as main memory. DRAM provides low latency and high throughput, making it suitable for continuously changing program data.

NAND Flash is non-volatile, meaning it retains information after power is removed. Although its latency is much higher than DRAM, it offers high storage density and lower cost per bit. Modern SSDs use multiple NAND chips and controllers to achieve high bandwidth and throughput.

Hierarchical placement:
DRAM is suitable for the main-memory level, while NAND Flash is suitable for secondary storage. A typical hierarchy can therefore be represented as:

CPU → L1/L2/L3 Cache → DRAM → NAND Flash SSD

This hierarchy balances speed, capacity, cost, and persistence.

- Evaluate MRAM and RRAM technologies based on access time, throughput, energy efficiency, and endurance. Analyze their potential applications in future hierarchical memory systems and assess their performance advantages and limitations.

## 5. MRAM vs RRAM

| Characteristic | MRAM | RRAM |
| --- | --- | --- |
| Access Time | Very low; close to SRAM-class performance in suitable designs | Potentially very low |
| Throughput | High | High potential |
| Energy Efficiency | High, especially because it is non-volatile | Potentially very high |
| Endurance | Generally very high | Depends strongly on device technology |
| Non-volatile | Yes | Yes |
| Main Limitation | Manufacturing cost/density trade-offs | Variability, endurance and manufacturing challenges |
| Potential Use | Cache, embedded memory, persistent memory | Embedded memory, dense storage, future cache/persistent memory |

MRAM (Magnetoresistive RAM) stores information using magnetic states. It is non-volatile, provides fast access, and generally has high endurance. Because it can retain data without continuous power, it can reduce energy consumption associated with data retention. MRAM is particularly promising for embedded memory, processor cache alternatives, and persistent memory.

RRAM (Resistive RAM) stores information by changing the resistance of a material. It offers high density, non-volatility, and potentially low-power operation. Its ability to support very small memory cells makes it attractive for future high-density memory systems. However, issues such as device variability, resistance-state control, endurance, and manufacturing consistency remain important challenges.

## Code of Conduct

I LAKSHMI DEVI J (192524300) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the Student: _________________________

RUBRICS FOR CALCULATION

| Criteria | Weightage(%) | Level 4 – Exemplary (5 Marks) | Level 3 – Proficient (4 Marks) | Level 2 – Developing (2–3 Marks) | Level 1 – Beginning (0–1 Mark) |
| --- | --- | --- | --- | --- | --- |
| Scope of Items | 20% | Selects highly relevant items with clear scope and justification | Selects appropriate items with minor gaps | Selection is limited or partially relevant | Items are unclear or not relevant |
| Comparison Depth | 25% | Provides detailed and comprehensive comparison across multiple aspects | Provides adequate comparison with some depth | Limited comparison with few aspects | Very minimal or no comparison |
| Use of Evidence | 20% | Supports comparison with accurate data, examples, or references | Uses some supporting evidence with minor gaps | Limited or weak supporting evidence | No supporting evidence provided |
| Critical Thinking | 20% | Demonstrates strong critical thinking with clear interpretation and reasoning | Shows reasonable interpretation with minor gaps | Basic interpretation; lacks depth | No meaningful interpretation |
| Clarity of Presentation | 15% | Well-organized, clear, and logically structured presentation | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganized and difficult to understand |

Marks Evaluation:

| Q. No | Scope of Items (20) |  | Comparison Depth (25) |  | Use of Evidence (20) | Critical Thinking (20) | Clarity of Presentation (15) | Total (out of 100) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
| 2 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
| 3 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
| 4 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
| 5 | / 4 |  | / 4 |  | / 4 | / 4 | / 4 |  |
|  | / 20 |  | / 25 |  | / 20 | / 20 | / 15 | / 100 |

Course Coordinator  	 	              	 	 	Faculty Incharge